3GPP TSG CT Meeting #39 	CP-080034

- Puerto Vallarta, MEXICO, 5th – 7th March 2008

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.008 | CR | 1168 |  rev | - |  Current version: | 8.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | TMGI misalignment between figure and table text |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code:  | MBMS |  |  |  |  |  |  | Date:  |  | 26/02/08 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | There is a misalignment in subclause 10.5.1.4 between figure 10.5.4a and the text in table 10.5.4 describing the Mobile Identity information element, particularly the coding of the Mobile Network Code (MNC) in case only two digits are used and the use of the Temporary Mobile Group Identity IE as MBMS Service ID where the current text is misleading. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The misalignment between figure 10.5.4a and the text in table 10.5.4 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | There is a risk of erroneus implementations that would lead to interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 10.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

#### 10.5.1.4	Mobile Identity

The purpose of the Mobile Identity information element is to provide either the international mobile subscriber identity, IMSI, the temporary mobile subscriber identity, TMSI/P-TMSI, the international mobile equipment identity, IMEI, the international mobile equipment identity together with the software version number, IMEISV, or the temporary mobile group identity (TMGI), associated with the optional MBMS Session Identity.

The IMSI shall not exceed 15 digits, the TMSI/P-TMSI is 4 octets long, and the IMEI is composed of 15 digits, the IMEISV is 16 digits (see 3GPP TS 23.003 [10]). The TMGI is at maximum 6 octets long and is defined in subclause 10.5.6.13. The MBMS Session Identity, if included, is 1 octet long (see 3GPP TS 48.018 [86]).

For packet paging the network shall select the mobile identity type with the following priority:

1-	P-TMSI: The P-TMSI shall be used if it is available.

2-	IMSI: The IMSI shall be used in cases where no P-TMSI is available.

For MBMS (pre-)notification (see 3GPP TS 44.018 [84] and 3GPP TS 44.060 [76]) the network shall select the mobile identity type "TMGI and optional MBMS Session Identity".

NOTE 1:	The type of identity "TMGI and optional MBMS Session Identity" is only used by the MBMS (pre-)notification procedure in of A/Gb mode.

For all other transactions except emergency call establishment, emergency call re-establishment, mobile terminated call establishment, the identification procedure, the GMM identification procedure, the GMM authentication and ciphering procedure and the ciphering mode setting procedure, the mobile station and the network shall select the mobile identity type with the following priority:

1-	TMSI: The TMSI shall be used if it is available.

2-	IMSI: The IMSI shall be used in cases where no TMSI is available.

For mobile terminated call establishment the mobile station shall select the same mobile identity type as received from the network in the PAGING REQUEST message. In case of enhanced DTM CS establishment (see 3GPP TS 44.018 [84]) the mobile station shall select the mobile identity type with the following priority in the PAGING RESPONSE message:

1-	TMSI: The TMSI shall be used if it is available.

2-	IMSI: The IMSI shall be used in cases where no TMSI is available.

For emergency call establishment and re-establishment the mobile station shall select the mobile identity type with the following priority:

1-	TMSI: The TMSI shall be used if it is available and if the location update status is UPDATED, and the stored LAI is equal to the one received on the BCCH from the current serving cell.

2-	IMSI: The IMSI shall be used in cases where no TMSI is available or TMSI is available but either the update status is different from UPDATED, or the stored LAI is different from the one received on the BCCH from the current serving cell.

3-	IMEI: The IMEI shall be used in cases where no SIM/USIM is available or the SIM/USIM is considered as not valid by the mobile station or no IMSI or TMSI is available.

In the identification procedure and in the GMM identification procedure the mobile station shall select the mobile identity type which was requested by the network, if available. If the requested identity is not available, then the mobile station shall indicate the identity type "No Identity".

In the ciphering mode setting procedure and in the GMM authentication and ciphering procedure the mobile shall select the IMEISV.

The Mobile Identity information element is coded as shown in figure 10.5.4/3GPP TS 24.008 and table 10.5.4/3GPP TS 24.008.

The Mobile Identity is a type 4 information element with a minimum length of 3 octet and 11 octets length maximal. Further restriction on the length may be applied, e.g. number plans.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mobile Identity IEI |  |  |  |  |  |  | octet 1 |
| Length of mobile identity contents |  |  |  |  |  |  |  | octet 2 |
| Identity digit 1 |  |  |  | odd/ even indic | Type of identity |  |  | octet 3 |
| Identity digit p+1 |  |  |  | Identity digit p |  |  |  | octet 4* |

Figure 10.5.4/3GPP TS 24.008 Mobile Identity information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mobile Identity IEI |  |  |  |  |  |  | octet 1 |
| Length of Mobile Identity contents |  |  |  |  |  |  |  | octet 2 |
| 0 | 0 | MBMS Sess Id indic | MCC/MNC indic | odd/ even indic | Type of identity |  |  | octet 3 |
| spare |  |  |  |  |  |  |  |  |
| MBMS Service ID |  |  |  |  |  |  |  | octet 4 octet 5 |
|  |  |  |  |  |  |  |  | octet 6 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 6a* |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 6b* |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 6c* |
| MBMS Session Identity |  |  |  |  |  |  |  | octet 7* |

Figure 10.5.4a/3GPP TS 24.008: Mobile Identity information element for type of identity "TMGI and optional MBMS Session Identity"

Table 10.5.4/3GPP TS 24.008: Mobile Identity information element

| Type of identity (octet 3) Bits |  |  |  |
| --- | --- | --- | --- |
| 3 | 2 | 1 |  |
| 0 | 0 | 1 | IMSI |
| 0 | 1 | 0 | IMEI |
| 0 | 1 | 1 | IMEISV |
| 1 | 0 | 0 | TMSI/P-TMSI |
| 1 | 0 | 1 | TMGI and optional MBMS Session Identity |
| 0 | 0 | 0 | No Identity (note 1) |
| All other values are reserved. |  |  |  |
| Odd/even indication (octet 3) Bit |  |  |  |
| 4 |  |  |  |
| 0 |  |  | even number of identity digits and also when the TMSI/P-TMSI or TMGI and optional MBMS Session Identity is used |
| 1 |  |  | odd number of identity digits |
| Identity digits (octet 3 etc) For the IMSI, IMEI and IMEISV this field is coded using BCD coding. If the number of identity digits is even then bits 5 to 8 of the last octet shall be filled with an end mark coded as "1111". For Type of identity "No Identity", the Identity digit bits shall be encoded with all 0s and the Length of mobile identity contents parameter shall be set to one of the following values: - “1” if the identification procedure is used (see subclause 9.2.11); - “3” if the GMM identification procedure is used (see subclause 9.4.13) If the mobile identity is the TMSI/P-TMSI then bits 5 to 8 of octet 3 are coded as "1111" and bit 8 of octet4 is the most significant bit and bit 1 of the last octet the least significant bit. The coding of the TMSI/P-TMSI is left open for each administration. For type of identity "TMGI and optional MBMS Session Identity" the coding of octet 3 etc is as follows: |  |  |  |
| MCC/MNC indication (octet 3) Bit |  |  |  |
| 5 |  |  |  |
| 0 |  |  | MCC/MNC is not present |
| 1 |  |  | MCC/MNC is present |
| MBMS Session Identity indication (octet 3) Bit |  |  |  |
| 6 |  |  |  |
| 0 |  |  | MBMS Session Identity is not present |
| 1 |  |  | MBMS Session Identity is present |
| MBMS Service ID (octet 4, 5 and 6) The contents of the MBMS Service ID field are coded as octets 3 to 5 of the Temporary Mobile Group Identity IE in Figure 10.5.154/3GPP TS 24.008. Therefore, bit 8 of octet 4 is the most significant bit and bit 1 of octet 6 the least significant bit. The coding of the MBMS Service ID is the responsibility of each administration. Coding using full hexadecimal representation may be used. The MBMS Service ID consists of 3 octets. |  |  |  |
| MCC, Mobile country code (octet 6a, octet 6b bits 1 to 4) The MCC field is coded as in ITU-T Rec. E.212, Annex A. |  |  |  |
| MNC, Mobile network code (octet 6b bits 5 to 8, octet 6c) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet 6b shall be coded as "1111". The contents of the MCC and MNC digits are coded as octets 6 to 8 of the Temporary Mobile Group Identity IE in Figure 10.5.154/3GPP TS 24.008. |  |  |  |
| MBMS Session Identity (octet 7) The MBMS Session Identity field is encoded as the value part of the MBMS Session Identity IE as specified in 3GPP TS 48.018 [86]. |  |  |  |
| NOTE 1: This can be used in the case when a fill paging message without any valid identity has to be sent on the paging subchannel and when the requested identity is not available at the mobile station during the identity request procedure. |  |  |  |